Revised: 08/09/2005

S3-200 (561-044-1)

Definitions: A failure is any unwanted response or non-response of the board while being tested according to Digilent test procedures. A failure is to be properly documented and reported to the repair technician by following the product test failure report procedures (Memo Number 581-019).

Project Environment:

ISE/Project Navigator Version: 6.3.03i

Project Functions and Results

- Verify FPGA programming success.

  - Press “PROG” button to reset testing program.

  - Check that “DONE” and “POWER LD8” are lit and remain lit at all times.

  - Make sure that the 7-segment display shows “12.00”.

  - Confirm that the “.” to the right of the “2” flashes each second.

  - Check that the 2 digits to the right of the “.” indicate minutes.

  - Check that the 2 digits to the left of the “.” indicate hours.

  - The clock time and alarm can be set using HyperTerminal.

  - Full functionality can be obtained through instructions found in PicoBlaze_clock_andPCB_Monitor.PDF

If the user does not receive the desired results from any of the above steps they are to record the fault on a Board Failure Report and give the board and report to the repair technician by following the product test failure report procedures (Memo Number 581-019).