Revised: 02/10/2005

S3-200 Test Procedures 503-044 (Rev E)

Preparing the Board for Testing

- Connect the 6pin JTAG cable.

- Connect the 5 VDC power adapter (LED2 is lit).

- Open iMPACT software.

  - At the “Operation Mode Selection” screen, select “Configure Devices” and click “Next”.

  - At the “Configure Devices” screen, select “Boundary-Scan Mode” and click “Next”.

  - At the “Boundary-Scan Mode Selection” screen, select “Automatically connect to cable and identify Boundary-Scan chain” and click “Finish.

  - At the “Boundary-Scan Chain Contents Summary”, click “OK”.

- Programming the FPGA.

  - At the “Assign New Configuration File” click “Bypass”.

  - At the second “Assign New Configuration File” select “new_Untitled.mcs” and click “Open”.

  - Right click on the right Xilinx chip icon and select “Program”.

  - At the “Program Options” window: ensure the “Verify” box is unchecked and click “OK”.

Program Operation

- Verify FPGA programming success.

  - Press “PROG” button to reset testing program.

  - “DONE” and “POWER LD8” are lit and remain lit at all times.

  - The 7-segment display shows “8.8.8.8.”.

  - Pressing “BTN3” turns off the display.

  - “BTN0” through “BTN2” and “SW0” through “SW7” have no functionality.